Project: Creating an IPsec site to site VPN tunnel using Packet Tracer

## Project Overview

The purpose of this project it to design and implement a secure , multisite network using Cisco Packet .The project covers the creation of a network topology , configuration of IP addressing on all devices , and implementation of a dynamic routing protocol.

In addition to standard connectivity , the project focuses on establishing a secure IPsec VPN between sites. Security Configuration include enabling the security technology package , creating extended ACLs to control access and configuring Internet Key Exchange Phase 1 and 2 policies to create secure IPsec tunnels. A crypto map is applied to router interfaces to ensure encrypted traffic between sites. All configurations will be validated by testing inter site communication to ensure that data can be securely traverse the VPN while meeting the access control requirements

Requirements

Create network Topology and label interface addresses and Site names

Configure OSPF for routing between the WAN interfaces  and Test communication

Configure IPSEC VPN

- Enable security technology package

- Configure an Extended ACL permitting the target on each router

- Configure the IKE Phase 1 ISAKMP policy on each router

- Configure the IKE Phase 2 IPsec policy on each router

- Configure the crypto map on the outgoing interface

## Structure

| Devices |  |  |
| --- | --- | --- |
| Router | Switch | End Devices |
| 2911 (3x) | 2960 (2x) | PC-PT (4x) |

Modules for Routers

HWIC-2T added to each router for Serial WAN connectivity

Address Plan

WAN IP Addressing

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| Site A Router | 10.10.10.2 | 255.255.255.248 |
| Site B Router | 20.20.20.2 | 255.255.255.248 |
| ISP Router | 10.10.10.1 | 255.255.255.248 |
| ISP Router | 20.20.20.1 | 255.255.255.248 |

LAN IP Addressing

| Site | Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| Site A | Router | Gig0/0 | 192.168.10.1 | 255.255.255.224 |
| Site A | PC1 | FE0/1 | 192.168.10.2 | 255.255.255.224 |
| Site A | PC2 | FE0/2 | 192.168.10.3 | 255.255.255.224 |
| Site B | Router | G0/0 | 172.168.0.1 | 255.255.255.224 |
| Site B | PC1 | FE0/1 | 172.168.0.2 | 255.255.255.224 |
| Site B | PC2 | FE0/2 | 172.168.0.3 | 255.255.255.224 |

## Configurations

Adding HWIC-2T Module

This is done all on three routers

Topology

OSPF Configuration

Site A

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf 1

Router(config-router)#router-id 2.1.2.1

Router(config-router)#network 10.10.10.0 0.0.0.7 area 0

Router(config-router)#network 192.168.10.0 0.0.0.31 area 0

Router(config-router)#exit

ISP OSPF Configuration

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf 1

Router(config-router)#router-id 3.1.3.1

Router(config-router)#network 10.10.10.0 0.0.0.7 area 0

Router(config-router)#network

00:24:09: %OSPF-5-ADJCHG: Process 1, Nbr 2.1.2.1 on Serial0/3/0 from LOADING to FULL, Loading Do

% Incomplete command.

Router(config-router)#network 20.20.20.0 0.0.0.7 area 0

Router(config-router)#exit

Site B OSPF configuration

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf 1

Router(config-router)#router-id

% Incomplete command.

Router(config-router)#router-id 4.1.4.1

Router(config-router)#network 172.168.0.0 0.0.0.31 area 0

Router(config-router)#network 20.20.20.0 0.0.0.7 area 0

Router(config-router)#exit

Testing Connectivity between the two sites

From Site A

Ping 172.168.0.2

From Site B

Ping 192.168.10.2

Configuring IPSEC VPN

Enabling security technology package on Site A and Site B Router

Site A Router

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#license boot module c2900 technology-package securityk9

Site B  Router

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#license boot module c2900 technology-package securityk9

Verification  security technology package enabled on Site A and Site B Router

Router#show version

Configuring Extended ACL

Site A Router

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#access-list 130 permit 192.168.10.0 0.0.0.31 172.168.0.0 0.0.0.31

^

% Invalid input detected at '^' marker.

Router(config)#access-list 130 permit ip 192.168.10.0 0.0.0.31 172.168.0.0 0.0.0.31

Router(config)#exit

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#

Site B Router

Router>

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#access-list 130 permit ip 172.168.0.0 0.0.0.31 192.168.10.0 0.0.0.31

Router(config)#exit

Router#

%SYS-5-CONFIG_I: Configured from console by console

Configuring  IKE Phase 1 ISAKMP Policy on Site A and B routers

Site A Router

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#crypto isakmp policy 10

Router(config-isakmp)#encryption aes 256

Router(config-isakmp)#authentication pre-share

Router(config-isakmp)#group 5

Router(config-isakmp)#exit

Router(config)#crypto isakmp key tm45 address 20.20.20.2

Router(config)#do wr

Building configuration...

[OK]

Router(config)#

Site B Router

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#crypto isakmp policy 10

Router(config-isakmp)#encryption aes 256

Router(config-isakmp)#authentication pre-share

Router(config-isakmp)#group 5

Router(config-isakmp)#exit

Router(config)#crypto isakmo key tm45 address 10.10.10.2

^

% Invalid input detected at '^' marker.

Router(config)#crypto isakmp key tm45 address 10.10.10.2

Router(config)#do wr

Building configuration...

[OK]

Router(config)#

Configuring IKE Phase 2 IPsec Policy

Site A router

Router(config)#crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

Router(config)#crypto map VPN-MAP 10 ipsec-isakpm

^

% Invalid input detected at '^' marker.

Router(config)#crypto map VPN-MAP 10 ipsec-isakpm

^

% Invalid input detected at '^' marker.

Router(config)#crypto map VPN-MAP 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

Router(config-crypto-map)#

Router(config-crypto-map)#

Router(config-crypto-map)#description this VPN connects Site B branch

Router(config-crypto-map)#set peer 20.20.20.2

Router(config-crypto-map)#set transform-set VPN-SET

Router(config-crypto-map)#match address 130

Router(config-crypto-map)#exit

Router(config)#do wr

Building configuration...

[OK]

Site B Router

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

Router(config)#crypto map VPN-MAP 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

Router(config-crypto-map)#description This connects to site B

Router(config-crypto-map)#set peer 10.10.10.2

Router(config-crypto-map)#set transform-set VPN-SET

Router(config-crypto-map)#match address 130

Router(config-crypto-map)#exit

Router(config)#do wr

Building configuration...

[OK]

Router(config)#

Configuring Crypto Map

Site A Router

Router(config)#int se0/3/0

Router(config-if)#crypto map VPN-MAP

*Jan 3 07:16:26.785: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

Router(config-if)#do wr

Building configuration...

[OK]

Router(config-if)#

Router(config-if)#

Site B Router

Router(config)#int se0/3/0

Router(config-if)#crypto map VPN-MAP

*Jan 3 07:16:26.785: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

Router(config-if)#exit

Router(config)#do wr

Building configuration...

[OK]

Router(config)#

Verification of VPN working

Pinging Site B Pcs from Site A Branch

Verification of VPN configuration working as intended

Router>en

Router#sh crypto ipsec sa